Types of Insulation

Wall Insulation; Internal Insulation (Dry Lining)

During the 1970’s, 80’s and 90’s “dry lining” involved covering over damp mildewed walls with sheets of plasterboard which were nailed onto wooden battens.  Nowadays “thermal liners” or “warmboards” are used; these are factory insulated sheets of plasterboard having a layer of high performance foam insulation bonded to the plasterboard. Usually the walls being upgraded now are not damp at all, but simply require better thermal insulation, though the term “dry lining” is still used.

When fitted to cold or damp walls thermal liners will dramatically improve heat retention.  The thermal mass of the dwelling will at the same time be lowered.

There are some minor disadvantages:

- Rooms will be made slightly smaller, (by the thickness of the thermal liners.)

- It may not be possible to insulate the reveals around windows, or the windows may look a bit odd from the inside because no window frame is visible.

- If the internal walls between rooms are constructed of solid masonry, then heat will be transferred to the external walls at the junctions where a room dividing wall meets an external wall. This is an example of cold bridging. These junctions, will be susceptible to condensation and mildew growth, unless the room dividing walls are also dry lined for some distance.

- Special fixings may be required when hanging pictures or shelves on a dry lined wall in order to get adequate support.

Wall Insulation; External Insulation

External insulation involves wrapping the entire outside wall of the building with insulation.  Normally expanded polystyrene or polyurethane boards are used.  The insulation then has to be protected from the weather so a thin layer of cement render or some external finish is applied.  This technique is relatively new to Ireland although it has been used very successfully in “Passive” and other houses in continental Europe.  It has the potential to upgrade a property from having the worst insulated walls to the best insulated walls in one step. However, external insulation may change the whole appearance of your house.  If it was a brick house, then it will no longer appear to be brick.  On the other hand, if a low maintenance finish is applied, you may never need to get the house painted again. It is best to check with your local authority to find out whether planning permission is required before changing the appearance of your house.

- There are some minor disadvantages:

- It is expensive.

- The outside walls of the house will be extended outwards by the thickness of the insulation, around 150mm. In the case of a semi-detached house this might look odd if the other house was unchanged. If the neighbours can be persuaded to join in and get the pair of houses insulated, then the insulation will be better and the price cheaper.

- The window frames and sills may be partially obscured when viewed from the outside and may look odd. If the windows are substandard single glazed, consideration should be given to replacing them and possible enlarging the openings as part of the process.

- The external rendering on the outside is thin.  It could possibly be dented by a bad driver parking a car or it could start to peel off if not properly applied after a few years.

- It may be difficult to find an experienced contractor for this work.

External insulation is the most effective way to upgrade an old building. As the Building Regulation standards become ever more demanding, external insulation is likely to become the only method whereby the owners of older properties can upgrade to the same standard as new homes being built.

Wall Insulation; Cavity Wall Insulation (retro fit)

Granular beads of expanded polystyrene or fibreglass are blown or injected into cavity walls.  Unfortunately most Irish dwellings are constructed of either

(a) Cavity walls already containing some insulation, or

(b) Hollow block.

The technique has limited value in both of these situations. It is most beneficial where there is a clear cavity but no insulation, such as with brick-faced concrete block properties built prior to 1977.

Roof Insulation

Cold Attic Layers of fibreglass wool or a granular material are laid between the joists.  Nowadays another layer is required on top of the joists and first layer. Insulating an attic which had little or no insulation is relatively inexpensive but has huge benefits. The attic space remains cold.

Pitched Roof Insulation (attic conversion) Insulation is placed between the rafters.  Another layer of foam insulation bonded to plasterboard is fixed onto the rafters.  The area requiring insulation is larger and will be much more expensive, but potentially a useable warm room is created.

Thermal Mass

Thermal mass is the ability of the dwelling to absorb heat and then slowly it release it over a period of time, about two or three days.

The internal walls of a house with a high thermal mass will typically feel very solid, whereas those in a low thermal mass house will appear to be light or hollow.

Concrete built houses without dry lining have high thermal mass.

Timber frame houses usually have low thermal mass.

Both types can have either good or bad insulation.

Advantages of High Thermal Mass In the spring and autumn chilly nights can follow warm sunny days.  You may be able to take advantage of the warmth stored up during the day by not turning on the heating system at all in the evening.  This is especially effective where there are south facing windows.

It is usually easier to fix pictures and shelves to the walls.

The internal temperature of the dwelling tends to remain stable. High thermal mass often suits a situation where there are people at home during the day.

Advantages of Low Thermal Mass

When you come home from work in the evening, the house will heat up very quickly, within about ½ hour, so the heating will not need to be on for long.  If the house is unoccupied during the day and therefore unheated, overall fuel bills will be lower.

Condensing Boiler

A condensing boiler incorporates a heat exchanger in the flue pipe which extracts heat from the exhaust gases.  This gives it an efficiency of more than 90%, as opposed to about 65% for an older 1980’s standard model.  The Building Regulations require that only condensing boilers be fitted nowadays.

Further energy savings can be achieved by situating the boiler inside the dwelling and not outside in a boiler house.  The boiler can often be situated in a kitchen or utility room.  A balanced flue is used in this situation which ensures that no exhaust gases of any kind can get into the dwelling.

By combining the above upgrade with efficient heating controls it should be possible to cut the fuel bill in half.

Wood Gasification Boiler

This burns fuel (usually logs) at a higher temperature than the normal wood stove with a back boiler, allowing for a more complete combustion of gases. It is therefore more efficient and less smoky. A normal wood stove should be seen as a replacement for an open fire in the living room, whereas a wood gasification boiler is a replacement for an oil fired boiler in a utility area.

Heating Controls

The best heating controls have separate time and temperature control for hot water and space heating.  Time control means they are each on a timeswitch. Temperature control means that there are thermostats, so that the burner in the boiler shuts down immediately when both the air temperature of the dwelling and the domestic hot water (DHW) temperatures are sufficient.

In any event the hot water should be on a separate zone, so that the boiler can be utilised for DHW during warm weather without heating the radiators.

When all these arrangements are in place, it is called boiler interlock. The interlock is not an actual device. This can be achieved in various ways; by having an advanced digital programmer and thermostatic radiator valves (TRV’s) with a bypass valve or by the right combination of simple time switches and thermostats controlling the heating zones.

Householders need not fully understand the interactions of various heating controls, but they should ensure that they know the whereabouts of both the time and the temperature controllers and how to operate both.  Then they should reduce the space heating control (room thermostat and/or TRV’s) to a level which still provides sufficient comfort.  At this point the optimum energy efficiency is achieved.

If your boiler has a tendency to “cycle” (i.e. it fires up for a short period of time, perhaps only a minute or two, and then shuts down again, and keeps on repeating this every twenty minutes or so) then that is an indication that you could benefit from better controls.  It is the equivalent of having your car start up by itself every half hour and idling for 2 minutes without going anywhere.

Heating Zones Most central heating installations are subdivided into separate zones. The heat supply to each zone can be turned on or off independently.

In a typical 2-zone installation the radiators are on a different zone to the DHW .This is a basic arrangement.

In a 3- zone installation the upstairs or bedroom radiators are on a separate zone. This allows them to be left unheated during the day which can cut fuel bills.

Larger houses can have more zones dividing various areas.

Motorised valves, thermostats, timeswitches and pumps control the flow of heat to the different zones.

T.R.V.’s or thermostatic radiator valves gradually restrict the heat supply to the radiator when the local air temperature reaches a set level. This set level can easily be adjusted by twisting the TRV. The effect of the TRV is to turn off the radiator automatically when the room gets warm.

In theory each room behaves like a separate zone with individual temperature control. In practice it will often be found more economical to supply quick bursts of heat at full flow to the radiators at set time periods throughout the day. This avoids the problem of having the boiler continue to pump heat wastefully around the pipework when most of the TRV’s have closed or are almost closed.

If you are installing a few TRV’s (maybe three to avail of the grant) put them on the radiators where you do not need heat so much. For example put them in bedrooms if they are not already on a separate zone, or in a spare room.

Cold Bridging

Cold bridging is where heat finds a path around the insulation to the outside. An example would be where a piece of insulation is missing. Other examples would be where a concrete window cill forms a continuous bridge from the inside to the outside of a cavity wall. Or the metal handles of well insulated French Doors. The affected area is a cold spot and likely to attract condensation. The amount of cold bridging in a building is mainly determined by techniques and good practice during construction, but can be alleviated afterwards to some extent, for example by checking that attic insulation covers the entire ceiling area.

Calculating Energy Costs

How can I easily calculate the running costs of an electrical appliance?

To calculate the cost in € cent per hour of running any electrical device, simply multiply its wattage by 0.018

You may find that the resultant cost is actually quite low.  The fact is that most electrical gadgets are very good value when their convenience is measured against running costs.  It’s only when you leave them switched on for too long that the cost mounts up. Electrical heaters are the usual reason for very high electricity bills.

Example.   (linked from ”a very cost effective improvement)

- A 300 watt outdoor security floodlight incorporating a PIR passive infra red detector.  Average “on” time is ½ hr per night. 300 x 0.018 = 5.4  So running cost is 5.4 cent/hour of “on” time. This adds up to19 cent per week or €10 per year.

- The same security light malfunctions or is incorrectly set up and stays on all night. (12 hours) It is again costing 5.4 cent/hour but is on for 84 hrs per week instead of 3½ hrs. This adds up to €4.54 per week or €236 per year.

Here is a link to a useful home appliance cost calculator   https://www.esb.ie/esbcustomersupply/residential/energy_efficiency/appliance_calc/appliance_calculator.jsp